Raspberry Pi Hub

Storage Write Analysis & SSD Recommendation

Awesom Living  |  September 2026  |  For Client Review

# 1. Purpose

This document quantifies the disk write operations generated by all devices and services running on the Raspberry Pi hub, and recommends the optimal storage configuration to eliminate the recurring SD card corruption issue.

# 2. Write Operations — All Devices & Services

When all 6 devices are paired and operating simultaneously, the following write activity occurs on the Pi's storage:

## 2.1 Zigbee Device Events (per day)

| Device | Events/Day | Writes/Event | Total Writes/Day | Data/Day |
| --- | --- | --- | --- | --- |
| Motion Sensor (PIR) | 150–300 | 5–8 | 750–2,400 | 2–6 MB |
| Door/Window Sensor x3 | 30–90 | 5–8 | 150–720 | 0.5–2 MB |
| Emergency Button | 0–5 | 5–8 | 0–40 | <0.1 MB |
| GLK Vital Tracker | ~1,440 (1/min) | 2–3 | 2,880–4,320 | 3–5 MB |
| CP Plus Camera (go2rtc) | 0 (in RAM) | 0 | 0 | 0 MB |
| AltumView (cloud only) | 0 | 0 | 0 | 0 MB |

Device event subtotal: ~3,780–7,480 write operations/day from sensor events.

## 2.2 Service-Level Writes (per day)

| Service | What It Writes | Volume/Day |
| --- | --- | --- |
| Zigbee2MQTT | state.json (every state change), device DB, log files | 10–30 MB |
| Mosquitto (MQTT) | Message persistence, connection logs | 5–10 MB |
| Node-RED | Flow context storage, event logs | 2–5 MB |
| PM2 (Process Manager) | stdout/stderr logs for all managed processes | 10–25 MB |
| Pi Dashboard (Express) | API request logs, error logs | 3–8 MB |
| Systemd Journal | System service logs (NetworkManager, kernel, etc.) | 5–15 MB |
| Cloudflared Tunnel | Connection state, reconnection logs | 1–3 MB |
| go2rtc | Minimal transcoding logs | 0.5–1 MB |
| OS Housekeeping | tmpfiles, cron, logrotate, apt cache | 2–5 MB |

Service write subtotal: ~38–102 MB/day of log and state data.

## 2.3 Total Daily Write Summary

| Metric | Normal Day | Heavy Activity Day |
| --- | --- | --- |
| Total write operations | ~7,000–10,000 | ~12,000–15,000 |
| Total data written | ~45–70 MB | ~80–115 MB |
| Peak IOPS | 2–5 IOPS | 8–15 IOPS |
| Annual data written | ~16–25 GB/year | ~29–42 GB/year |

Key Point: The write volume is modest (~45–115 MB/day). The problem is not volume — it is the pattern: thousands of tiny random writes (state.json updates, log entries) which are the worst case for SD cards, and the SDIO bus vulnerability that corrupts the card on WiFi driver crashes.

# 3. Why SD Cards Fail Under This Workload

| Factor | SD Card | SSD |
| --- | --- | --- |
| Write unit | Erases 128–512 KB block for 1-byte write | 4 KB page writes, block-level wear levelling |
| Write amplification | 10x–50x | 1x–3x |
| Wear levelling | Basic or none | Sophisticated controller with over-provisioning |
| Endurance (P/E cycles) | 500–3,000 | 3,000–100,000 |
| Power-loss protection | None | Capacitor-backed flush on many models |
| Random write speed | 0.1–1 MB/s | 50–200 MB/s |
| TBW rating | Unrated (~10–30 TB est.) | 60–600 TBW (manufacturer guaranteed) |

At our write rate (~30 GB/year), an SD card lasts 1–3 years. An SSD rated at 60 TBW would last 2,000+ years — effectively infinite.

# 4. Storage Options Compared

## Option A: SD Card (Read-Only OS) + USB SSD (All Writes)

The SD card holds the OS in read-only mode (overlayfs). All write-heavy operations — logs, Zigbee2MQTT state, MQTT persistence, PM2 logs — are redirected to a USB SSD.

| Aspect | Assessment |
| --- | --- |
| SD card protection | Good — mounted read-only, immune to write corruption |
| SDIO bus risk | STILL PRESENT — onboard WiFi still uses SDIO. A WiFi firmware crash can still lock the bus and corrupt the SD card's read-only metadata. |
| Setup complexity | HIGH — overlayfs config, 8+ mount-point redirections, custom fstab |
| Maintenance | HIGH — OS updates require disabling read-only mode, re-enabling after |
| Failure mode | SSD fails: services lose state. SD fails: system won't boot. |
| Cost | ~INR 1,500–2,500 (SSD) + existing SD card |
| Script changes | Major rewrite needed |

Risk: The SDIO bus vulnerability still exists. Even with a read-only SD card, a WiFi firmware crash can lock the SDIO controller, potentially corrupting the card's partition table or filesystem metadata.

## Option B: SD Card (OS) + Pi RAM / tmpfs (All Writes)

The SD card holds the OS normally. All write-heavy paths (/var/log, Zigbee2MQTT state, MQTT persistence, PM2 logs) are mounted as tmpfs — writing to RAM instead of disk. No additional hardware needed.

| Aspect | Assessment |
| --- | --- |
| SD card protection | Good — near-zero writes to SD card during normal operation |
| SDIO bus risk | STILL PRESENT — same WiFi/SDIO vulnerability as Option A |
| Setup complexity | MEDIUM — tmpfs mounts in fstab, symlinks for service data dirs |
| Data persistence | NONE — all logs, Zigbee device state, MQTT data lost on every reboot or power cycle. Devices may need re-pairing after crash. |
| RAM usage | HIGH — Pi 4 has 1–4 GB total RAM. Services + tmpfs together could exhaust memory, causing OOM kills and instability. |
| Maintenance | MEDIUM — no OS update issues, but lost state means manual recovery after any reboot |
| Failure mode | Any reboot (even clean) wipes all runtime state. Zigbee2MQTT loses device database — sensors may need re-pairing. |
| Cost | INR 0 (no additional hardware) |
| Script changes | Moderate — add tmpfs mounts, adjust service data paths |

Critical Risk: Zigbee2MQTT stores its device database and network coordinator state in /opt/zigbee2mqtt/data/. If this is on tmpfs, every reboot wipes all paired devices — the resident's sensors stop working until someone manually re-pairs them. For a remote-monitored elderly care system, this is unacceptable.

## Option C: USB SSD (OS + All Write Operations) — Recommended

The entire operating system and all data live on a single USB SSD. No SD card at all. Raspberry Pi 4 natively supports USB boot.

| Aspect | Assessment |
| --- | --- |
| SD card protection | N/A — no SD card. Root cause eliminated entirely. |
| SDIO bus risk | ELIMINATED — OS is on USB, not SDIO. WiFi crash cannot affect boot drive. |
| Setup complexity | LOW — flash OS to SSD instead of SD card, one-time bootloader update |
| Data persistence | FULL — all data persists normally across reboots |
| Maintenance | LOW — behaves like a normal Pi, just faster. OS updates work normally. |
| Failure mode | SSD fails: full replacement. But SSD MTBF is 1.5–2 million hours. |
| Cost | ~INR 2,000–5,500 (120–250 GB SSD) |
| Script changes | NONE — same setup-pi-fresh.sh, just flashed to SSD |

Why This Wins: This is the only option that eliminates BOTH the SD card's write-endurance weakness AND the SDIO bus vulnerability. The OS runs on USB — completely isolated from the WiFi chipset's SDIO bus. Even if the WiFi firmware crashes, the boot drive is untouched.

# 5. Recommended SSDs (Available in India)

For our workload, any modern SSD is vastly over-specified. Selection criteria: compact form factor, USB 3.0, reliability, and availability in India.

## 5.1 Portable USB SSDs (Plug-and-Play)

| Model | Capacity | Speed | Price (INR) | Notes |
| --- | --- | --- | --- | --- |
| Samsung T7 Shield | 250 GB | 1,050 MB/s | 4,000–5,500 | IP65 dust/water. Best for homes. 3yr warranty. |
| Samsung T7 | 500 GB | 1,050 MB/s | 4,500–6,000 | Compact, reliable, widely available. |
| Kingston XS2000 | 500 GB | 2,000 MB/s | 4,500–5,500 | Coin-sized. USB 3.2 Gen 2x2. |
| SanDisk Extreme V2 | 500 GB | 1,050 MB/s | 5,000–6,500 | Rugged, IP55 rated. |

## 5.2 SATA SSD + USB Enclosure (Budget Option)

| Component | Model | Price (INR) |
| --- | --- | --- |
| SSD | Crucial BX500 240 GB (2.5" SATA) | 1,300–1,800 |
| Enclosure | UGREEN / Sabrent SATA-to-USB 3.0 | 500–800 |
| Total | — | 1,800–2,600 |

Best Pick: Samsung T7 Shield 250 GB (~INR 4,000–5,500) — compact, IP65 dust/water proof, 3-year warranty, plug-and-play. Budget alternative: Crucial BX500 + USB enclosure (~INR 2,000).

# 6. Endurance — How Long Will Each Option Last?

| Storage | Endurance | Lifespan at ~30 GB/yr | Lifespan at ~50 GB/yr |
| --- | --- | --- | --- |
| SD Card (64 GB) | ~10–30 TB (est.) | 1–3 years + corruption risk | 6 months–2 years |
| SD Card + RAM (tmpfs) | Near-zero writes | 5+ years (but data lost on reboot) | 5+ years |
| Samsung T7 (250 GB) | 150 TBW (warranted) | 5,000 years | 3,000 years |
| Crucial BX500 (240 GB) | 80 TBW (warranted) | 2,600 years | 1,600 years |

# 7. Final Comparison — All Options at a Glance

|  | A: SD(RO) + SSD | B: SD + RAM | C: SSD Only |
| --- | --- | --- | --- |
| Corruption risk | LOW (but SDIO still exposed) | LOW (but SDIO still exposed) | NONE |
| SDIO bus risk | Present | Present | Eliminated |
| Data on reboot | Preserved | LOST (sensors may need re-pair) | Preserved |
| Setup complexity | High | Medium | Low |
| Maintenance | High | Medium (manual recovery) | Zero |
| RAM impact | None | High (200–500 MB used) | None |
| Additional cost | INR 2,000–5,500 | INR 0 | INR 2,000–5,500 |
| Script changes | Major rewrite | Moderate | None |
| Recommended | No | No | Yes |

Final Recommendation: Option C — Boot the Pi entirely from a USB SSD (Samsung T7 Shield 250 GB). This eliminates SD card corruption, the SDIO bus vulnerability, and requires zero changes to the existing setup script. Total investment: ~INR 4,000–5,500 per hub.

# 8. Implementation Steps (30–45 Minutes)

Step 1: Update Pi 4 bootloader for USB boot (one-time, 2 minutes via SD card).

Step 2: Flash Raspberry Pi OS to USB SSD using Raspberry Pi Imager.

Step 3: Connect SSD to Pi USB 3.0 port (blue). Remove SD card.

Step 4: Power on — Pi boots from SSD. Run existing setup-pi-fresh.sh (no changes needed).

Step 5: Verify all services. Dashboard, sensors, camera stream — all work identically.

Prepared by: Backend Team  |  Awesom Living  |  VAYUZ Technologies, Noida