# Awesom Living

Dashboard Data Calculation Guide

How the app calculates and displays sleep & health data

Date: September 2026  |  Version: 1.0

## 1. Introduction

The Awesom Living app has two main sleep-related screens for the family to monitor their parent's sleep and health:

Dashboard Summary — The Dashboard Summary is the first screen the family sees after opening the app. It provides a quick overview of the most recent sleep session along with heart rate, activity, and sleep stages.

Dashboard Details — The Dashboard Details screen shows deeper information including breathing rate, time spent in bed, sleep quality rating, and weekly comparisons.

Both screens pull data from the GLK bed sensor placed under the mattress. This document explains, in plain language, what each data point means and how it is calculated.

## 2. Data Source

All sleep and health data comes from the GLK Vital Tracker — a thin sensor strip placed under the mattress. It continuously monitors whether someone is in bed, their heart rate, breathing rate, and body movement. The sensor sends readings to the backend approximately every 30 seconds.

The backend receives two types of readings from the sensor:

Status readings — These are status updates sent approximately every 30 seconds. They tell us whether the person is in bed, their current heart rate, breathing rate, and movement level.

Sleep assessment readings — These are the sensor's own assessment of whether the person is actually sleeping or just lying awake. The sensor analyses patterns in heart rate and movement to determine this.

## 3. Sleep Sessions

A "sleep session" is one continuous period of sleep. The system detects when sleep starts, when it ends, and handles brief awakenings during the night.

### How a Sleep Session Starts

A sleep session begins the moment the sensor's sleep assessment indicates the person has fallen asleep. Simply lying in bed without sleeping does not start a session.

### How a Sleep Session Ends

A sleep session ends when the person has been awake (or out of bed) continuously for 15 minutes. This 15-minute rule is important — if someone wakes up briefly (to use the bathroom, drink water, or just stir) but falls back asleep within 15 minutes, the system considers it part of the same sleep session, not a separate one.

Example: If a person falls asleep at 10:00 PM, wakes briefly at 2:00 AM for 5 minutes, then falls back asleep — this is counted as one continuous session, not two separate ones.

### Merging Short Gaps

Even after the 15-minute rule, there can be situations where two sessions end up very close to each other (within 30 minutes). The system automatically merges such sessions into one, treating them as a single sleep block. This prevents the night from being split into many small fragments due to brief awakenings.

### Minimum Duration

Any sleep session shorter than 15 minutes is ignored — it is treated as sensor noise or a very brief nap, not a real sleep period.

### Which Session Is Shown

Both the Dashboard Summary and Dashboard Details show data from the most recent (last) sleep session of the selected date. If a person had multiple sleep sessions in a day, only the latest one is displayed.

### Date Attribution

A sleep session is attributed to the calendar date (Indian Standard Time) that contains the majority of its duration. For example, if someone sleeps from 11:00 PM on September 23 to 7:00 AM on September 24, most of the sleep falls on September 24, so it appears under September 24 in the app.

### Sleep Validation

The system verifies that the sensor's own sleep assessment confirms actual sleep occurred during a session. If the sensor's assessment records show only "awake" throughout the session (for example, the person was just lying in bed without sleeping), that session is filtered out and not shown.

## 4. What Each Screen Shows

Here is a comparison of what data appears on each screen:

| Data Point | Dashboard Summary | Dashboard Details |
| --- | --- | --- |
| Sleep Start Time | Yes | Yes |
| Wake Time | Yes | Yes |
| Sleep Duration | Yes (hours) | Yes (hours) |
| Sleep Score | Yes (percentage) | Yes (percentage) |
| Heart Rate | Yes (average + graph) | Yes (average + graph) |
| Breathing Rate | No | Yes (average + graph) |
| Activity / Movement | Yes (intervals) | Yes (intervals) |
| Sleep Stage Timeline | Yes | Yes |
| Snoring | Yes (episodes + minutes) | Yes (episodes + minutes) |
| Restlessness | Yes (episodes + minutes) | Yes (episodes + minutes) |
| Currently Sleeping? | Yes (true/false) | No |
| Previous Session | Yes (start & end time) | Yes (start & end time) |
| Time in Bed | No | Yes (hours + intervals) |
| Sleep Quality Label | No | Yes (Good/Average/Poor) |
| Week-over-Week Trend | No | Yes (has previous week) |

## 5. Sleep Score

The sleep score is a percentage (0–100%) that indicates how much of the sleep session was spent actually sleeping versus being awake.

### How It Is Calculated

The sleep session is divided into 30-minute blocks. For each block, the system counts how many sensor readings show "sleeping" versus "awake." The percentage of sleeping readings in each block becomes that block's score. The final sleep score is the average of all these block scores.

Example: If there are 4 blocks in a session and their scores are 100%, 100%, 100%, and 98%, the average sleep score would be ~100% (99.5%, rounded to 100%).

This score is shown on both the Summary and Details screens, and it drives the sleep quality graph (the bar chart showing sleep percentage per 30-minute slot).

## 6. Sleep Quality Label

The Details screen shows a sleep quality rating based on the sleep score. This is a simple human-readable label:

"Good" — sleep score is 85% or higher.

"Average" — sleep score is between 70% and 84%.

"Poor" — sleep score is below 70%.

## 7. Heart Rate

The sensor measures the person's heart rate continuously while they are in bed. Heart rate values are only accepted if they fall within a valid range (30 to 220 beats per minute).

### How It Is Displayed

Heart rate readings from the most recent sleep session are grouped into 30-minute blocks. The average heart rate within each block is calculated. The overall average heart rate shown on the screen is the average of all these block averages.

Both screens show the average heart rate and a graph of heart rate over time during the session.

## 8. Breathing Rate (Details Screen Only)

The breathing rate (also called respiration rate) measures how many breaths the person takes per minute. Like heart rate, it is measured by the bed sensor during sleep.

Breathing rate is displayed on the Details screen only. It follows the same 30-minute block averaging as heart rate and shows both an overall average and a time-based graph.

## 9. Sleep Stage Timeline

The sleep stage timeline is a visual representation of how the person's sleep progressed through the night. It shows periods of sleep versus awake as reported by the bed sensor.

The sensor reports whether the person is "sleeping" or "awake" at each reading. The system groups consecutive same-stage readings into segments, each showing the start time, end time, and how long that stage lasted (in minutes).

Note: If there is a gap in sensor data (e.g., the sensor goes offline), the system closes the current segment and starts a fresh one after data resumes. This prevents incorrectly showing sleep during a period where no data was available.

## 10. Snoring Detection

The bed sensor detects vibration patterns associated with snoring. The system processes these raw detections to identify distinct snoring episodes.

### How Snoring Episodes Are Identified

Individual snoring readings are first smoothed using a sliding window — the system looks at a 5-minute window of readings, and if 3 or more readings in that window show snoring, the whole window is marked as a snoring cluster. This prevents counting every single 30-second snoring reading as a separate event.

These clusters are then grouped into episodes. Clusters that are close together in time are merged into a single episode. The total snoring time and number of distinct episodes are shown on both screens.

Snoring data is scoped to the most recent sleep session only, so the snoring duration will never exceed the total sleep duration.

## 11. Restlessness Detection

Restlessness measures how much the person moved or tossed and turned during sleep. It works similarly to snoring detection.

Movement readings are smoothed using the same sliding window approach (5-minute window, 3+ readings = restless cluster). Nearby clusters are merged into episodes. Only episodes lasting at least 1 minute are shown.

Like snoring, restlessness is scoped to the most recent sleep session and shown on both screens with episode count, total minutes, and individual episode times.

## 12. Activity / Movement Level

The activity metric shows the level of physical movement during the sleep session, measured on a scale from 0 (no movement) to 10 (high movement). A value above 5 indicates turning or significant movement.

Activity readings are grouped into intervals based on when the person was in bed. The average activity level across all readings in each interval is calculated and displayed.

Activity data is scoped to the sleep session window only, so daytime movement (like sitting on the bed during the afternoon) does not appear in the sleep activity chart.

## 13. Time in Bed (Details Screen Only)

The "Time in Bed" metric on the Details screen shows the total time the person spent physically lying in bed during the sleep session, regardless of whether they were sleeping or awake.

The sensor reports an "in bed" or "out of bed" status with each reading. The system identifies continuous periods of being in bed and merges very short out-of-bed blips (gaps under 5 minutes) — these are usually sensor noise rather than the person actually getting up.

The in-bed intervals are clipped to the sleep session window so only bed time during the actual sleep session is counted.

## 14. Currently Sleeping Flag (Summary Only)

The Dashboard Summary includes a true/false indicator showing whether the person is currently sleeping. This is determined by looking at the most recent sensor reading:

"True" — the person is currently in bed AND the sensor reports a sleeping state.

"False" — the person is either out of bed or awake.

This allows the family to see at a glance whether their parent is currently asleep.

## 15. Previous Sleep Session

Both screens show the start and end time of the previous sleep session (the most recent session from a prior date). This helps the family compare tonight's sleep with the last recorded sleep.

The previous session is identified from the same set of sensor data — it is the last valid session attributed to a date before the currently selected date. If no previous session is available in the current data window, the system looks it up from stored historical records.

## 16. Weekly Comparison (Details Screen Only)

The Details screen includes a flag indicating whether there is data from the previous week available for comparison. When available, the system calculates the average sleep score from the past 7 days to show trends.

## 17. Summary: How Data Flows

Here is a simplified view of how data moves from the sensor to the app:

1.  The GLK bed sensor continuously monitors the bed and sends readings every ~30 seconds to the Raspberry Pi hub in the home.

2.  The Raspberry Pi forwards these readings to the backend server.

3.  When the app requests data for a specific date, the backend pulls all sensor readings for a wide time window around that date.

4.  The backend identifies sleep sessions from these readings using the rules described in Section 3.

5.  For the most recent valid session, the backend calculates all metrics: sleep score, heart rate, breathing rate, snoring, restlessness, activity, and sleep stages.

6.  The calculated data is sent to the app, which displays it on the Summary or Details screen.

End of Document